Release Notes For OpenSim version 2.4

With the release of OpenSim version 2.4.0 the biggest change for users occurs in the data exported from Visual3d to OpenSim. Starting with OpenSim 3.2 Beta, OpenSim now looks for the ground reaction force data in a separate .xml or .mot file. Visual3d will now generate this new ground reaction force file on export (called Visual3_SIMM_grf.mot) but there are some consequences of the new OpenSim external force input methodology.

From a user’s point of view the real change occurs when they run the OpenSim RRA and CMC Tools. Now when a user loads the Visual3d_Setup_RRA.xml and Visual3d_SetUp_CMC.xml into the OpenSim, OpenSim will now look for an External Load file called Visual3d_SIMM_grf.mot. This file will be generated automatically by OpenSim; however Visual3d_SIMM_grf.mot.  does not work with the Visual3d export process. Thus when running both the RRA and CMC tools the users has to select the “Actuators and External Loads” tab and in the “External Loads Section” change the “External loads specification file” to Visual3d_SIMM_input_grf.xml. (The file, Visual3d_Simm_input_grf.xml, is supplied with the Visual3 to OpenSIm 2.4.0 distribution.)

The revised “Instruction for Running Visual3d toOpenSim Integration” section of this document gives the exact details on running the new OpenSIM RRA tool and the CMC Tools.

In addition to the changes related the external force export, there are three other revisions for exporting from Visual3d to OpenSim version 2.4:

- The torso and feet are now automatically scaled to be equal to the average of the left and right femoral lengths. (The new torso and feet manual scale factors are written to the Visual3d_Scale_ScaleSet.xml file.) This modification was made because most users build their feet without markers at the very end of the foot and at the top of the head and thus using the average thigh length seems a reasonable scaling approximation for these segments. (Physical anthropometry studies have shown thigh length is a somewhat strongly correlated with height)

- In the OpenSim Dialog the user is now prompted for the OpenSim version number. For export to OpenSim 2.4 select OpenSim Version “2.3 Beta and Newer ”.

- After running RRA in OpenSim version 2.4.0 sometimes the bones will disappear. (This is apparently happens because RRA will occasionally leave the “Working Directory”.) To fix the problem go to the OpenSim EditPreference pull-down menu and change the Geometry path to the folder where OpenSim installed the “Geometry” folder. In OpenSim 2.4 this is often C:/OpenSim2.4.0/Geometry.

Exporting Data From Visual3d to OpenSim

Preface

Visual3d has recently added a feature which will allow the user to export an OpenSim compatible .mot file which can be used as input to OpenSim gait simulations. The Visual3d to OpenSim integration is based on the OpenSim tutorial “Generating a Muscle-Actuated Simulation in OpenSim”.  It is assumed that the user is familiar with this tutorial and the files used in this process.  For details see:  simtk.org/home/opensim.

(Note: the default OpenSim model used in the Visual3d to OpenSim process is the model gait2392_simbody.osim which is distributed with the OpenSim software.)

Introduction

Visual3d 4.0 now allows the user to export OpenSim compatible motion files designed for use with OpenSim gait models. To accomplish this Visual3d creates an OpenSim compatible motion file, Visual3d_SIMM_input.mot, which bypasses the Scaling and Inverse Kinematics (IK) in OpenSim and can be used directly by the OpenSim Residual Reduction Algorithm.

Visual3d’s ability to write OpenSim compatible .mot files is based on Visual3d’s own Inverse Kinetics algorithm. The theory behind the Visual3d to OpenSim Inverse Kinematics is a three stage process:

- Visual3d computes the scale factors which will scale the OpenSim gait model to the Visual3d static calibration model. These scale factors get written to a special OpenSim scale set file named Visual3d_Scale_ScaleSet.xml.  (See the Visual3d Scaling section of this document for more details.)

- Visual3d uses Inverse Kinematics to fit the scaled OpenSim gait model to the Visual3d Static Calibration Model. (This is similar to the OpenSim Marker Placer Process).

- Visual3d then uses Inverse Kinematics to fit the gait data to the newly calibrated model.

Modeling Requirements

The Following model requirements must all be met. Please read the three requirements carefully:

- The Visual3d model used for OpenSim output must include the right foot (RFT), right shank (RSK), right thigh (RTH), left foot (LFT), left shank (RSK), left thigh (RTH), pelvis (RPV) and a Thorax/Ab (RTA).

- The Thorax/Ab (RTA) segment must be built in a very specific way. First, the proximal end of the RTA should be the created at the exact same location as the proximal end of the pelvis. Thus the proximal end of the RTA forms the joint with the pelvis while the distal end of the RTA should be built at approximately at the height of the shoulder shoulders. If constructed in this manner, the RTA segment will now have the Z axis (which goes distal to proximal) pointed downward and the Y axis pointed backward. This must be changed so the RTA segment coordinate system matches the local coordinate system of the other segments. To do this you go click the “Modify the Segment Coordinate System” button in the “Segment Properties” tab. Then set the “AP Axis” to –Y and the “Distal to Proximal” to be –Z.  Building the trunk in the exact manner described above will create a trunk whose origin is at the proximal end of the pelvis, its Z axis is vertical and its Y axis is Anterior. This coordinate system is needed to assure the trunk is properly oriented when you load the.mot file to OpenSim.

- One current limitation of the Visual3d to OpenSim approach is that in order to handle the motion of the pelvis relative to the LAB all subjects are assumed to be walking in the +Y direction, with the global +Z axis in vertical direction and the +X axis pointing to the right. If the user’s LAB coordinate system is not setup this way they can get around this limitation by creating “virtual” lab segment with this orientation. The virtual lab must be named v3d_lab. See Appendix for an example of how to create of Visual3d v3d_lab.

Visual3d and the OpenSim xml Files

As mentioned previously, the Visual3d to OpenSim process is designed to work with the OpenSim in the manor outlined in the OpenSim tutorial “Generating a Muscle-Actuated Simulation in OpenSim”. To work successfully the process required that special versions of several OpenSim xml input be created. Below is the list of new OpenSim xml files and how they were created by modifying the original files:

Visual3d_Setup_Scale.xml  (modified from subject01_Setup_Scale.xml)

- The <ScaleTool> name has been changed to Visual3d.

- The subject <mass> and <height> needs to be customized for each subject

- The <Marker Set> is no longer required and reference to this file has been removed

- <scaling_order> is set only to manualScale. (The manual scale factors for the segments will be written by Visual3d to the Visual3d_Scale_ScaleSet.xml file.)

- The <MeasurementSet> .xml file and its corresponding .trc file are no longer needed and reference to these files have been removed

- The <ModelScaler> section has been removed since this is now done by Visual3d

- The <MarkerPlacer> section has been removed since this is now done by Visual3d

Visual3d_Setup_RRA.xml (modified from subject01_Setup_RRA.xml)

- The <CMCTool> name has been changed to Visual3d_walk1_RRA

- The <model_file> name has been changed to Visual3d_simbody.osim.

- The <initial_time> and <final_time> for RRA have been set to 0.0 and 1.0 but this will be changed by the user when operating OpenSim to match the desired time range for RRA

- The <desired_kinematics_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the kinematics.

- The <external_loads_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

- The <external_loads_model_kinematics_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

Visual3d_Setup_CMC.xml  (modified from subject01_Setup_CMC.xml)

- The <CMCTool> name has been changed to Visual3d_walk1.

- The <model_file> has been changed to Visual3d_simbody_adjusted.osim.

- The <initial_time> and <final_time> for CMC have been set to 0.0 and 1.0 but this will be changed by the user when operating OpenSim to match the desired time range for CMC

- The <desired_kinematics_file> has been changed to: ResultsRRA/Visual3d_walk1_RRA_kinemtics_q.sto.

- The <external_loads_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

- The <external_loads_model_kinematics_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

Visual3d_Setup_Foward.xml  (modified from subject01_Setup_Forward.xml)

- The <ForwardTool> name has been changed to Visual3d_walk1

- The <model_file> name has been changed to Visual3d_simbody_adjusted.osim

- The <initial_time> and <final_time> for Forward simulation have been set to 0.0 and 1.0 0 but this will be changed by the user when operating OpenSim to match the desired time range for the Simulation

- The <controls_file>  has been changed to: ResultsCMC/Visual3d_walk1_controls.xml

- The <states_file  file> has been changed to: ResultsCMC/Visual3d_walk1_states.sto

- The <external_loads_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

- The <external_loads_model_kinematics_file> is now Visual3d_SIMM_input.mot. This is the name of the file that is written by Visual3d containing the fore platform information.

Visual3d_Scale_ScaleSet   (modified from subject01_Scale_ScaleSet.xml)

This file must be present so Visual3d can write the manual scale factors used for the pelvis, thigh and tibia segments.

Files No longer Required

Visual3d does manual scaling on all segments and does its own IK, thus many of the OpenSim files related to the Marker Set, Scaling and IK are no longer required.

Visual3d Scaling

Visual3d automatically scales the OpenSim model by:

- Changing the OpenSim scaling method to manual scaling for all segments. (The simplified version of the OpenSim scaling setup file, called Visual3d_Setup_Scale.xml , should be used to scale the model in OpenSim.)

- Updating the file Visual3d_Scale_ScaleSet.xml to include the Visual3d computed scale factors for the pelvis, left and right thigh and left and right tibia segments.

Important Note: The manual scale factors written to the Visual3d_Scale_ScaleSet.xml file must be used to scale the pelvis, thigh and tibia segments in OpenSim. If these scale factors are not used then center of pressure, which is also written by Visual3d to the Visual3d_SIMM_input.mot, will not be adjusted properly in relation to the model. (The scale factors for the feet and torso do not affect the relationship between the center of pressure.)

The manual scale factors computed by Visaul3d are based on the subject calibration data and the OpenSim segment geometry (as specified in the gait2392_simbody.osim). Specifically, the segments are scaled by:

Pelvis: To scale the pelvis Visual3d computes the distance (in meters) between the hip centers in the subject calibration and divides this value by 0.167. The value 0.167 is the distance between the medial and lateral hip location in the file gait2392_simbody.osim. (One important note is that by default OpenSim scales the pelvis based on the distance between the left and right ASIS; however since Visual3d does not require targets be placed on the left and right ASIS the pelvis is scaled based on the distance between the hip joint centers which will always be present.)

Thigh: To scale the thigh Visual3d computes the distance between the hip and knee joint center in the subject calibration and divides this value by 0.396. The value 0.396 is the distance between the hip and knee location in the file gait2392_simbody.osim. (Note: the knee location will vary with knee joint angle in OpenSim and the value 0.396 represents the knee location at zero degree of knee flexion.)

Shank: To scale the thigh Visual3d computes the distance between the knee and ankle joint center in the subject calibration and divides this value by 0.43. The value 0.43 is the distance between the knee and ankle location in the file gait2392_simbody.osim.

Torso, Calcaneus and Toes: Because most users build their feet without markers at the very end of the foot and at the top of the head and thus using the average thigh length is used to scale these segments. (Physical anthropometry studies have shown thigh length is a somewhat strongly correlated with height)

Instruction for Running Visual3d toOpenSim Integration

Part 1 – Creating the motion file for OpenSim

- Start Visual3d and create a Visual3d hybrid model containing the following segments:

right foot (RFT), right shank (RSK), right thigh (RTH),  left foot (LFT), left shank (RSK), left thigh (RTH), pelvis and a trunk (RTA)

- If the subject is not walking in the +Y with +Z as vertical create a v3d_lab segment as outlined in the Appenddix

- Go to File and select ExportExport OpenSim Motion File. Alternatively the user could export the file using ExportExport OpenSim Motion File without Cleanup. (The only difference between the two method is that if the method without Cleanup is selected then the OpenSim Inverse Kinematics remains applied to the model after export and the resulting position state variables get written to the “DERIVEDInverse Kinematics” folder of Visual3d “Signal and Events” data tree.)

- Enter the data range and click the OK button.

- Visual3d now needs the required Visual3d_Scale_ScaleSet.xml file and will bring up a File Selection Dialog to help the user make sure that Visual3d will update the correct file.

- After selecting the ScaleSet file, Visual3d now applies the Inverse Kinematics process to the gait data. (This can take several minutes.)

Once steps 1-6 are completed, Visual3d will have updated the Visual3d_Scale_ScaleSet.xml file and have created a Visual3d_SIMM_input.mot file which can be loaded into OpenSim.

Part 2 – Scaling and Viewing  the motion file for OpenSim

- Start OpenSim and load the file openSim  model. (This will be either gait2392_simbody.osim or gait2392_simbody.osim)

- From the Tools pulldown select Scale Model

- In the Scale Tool Dialog select Settings > Load Settings

- Open the file Visual3d_Setup_Scale.xml

- Uncheck the Adjust Model Markers box

- Hit the Run botton to create the scaled model . The model will be called Visual3d  in the Navigator window

- Left Click on model 3DGaitModel2354 in the navigator Window then right click and select Close.

- A dialog will appear – select No so you do not save settings.

- Vusial3d will now be the only Model

- From the File pull-down select Load Motion and load Visual3d_SIMM_input.mot

- Hit the play button to Animate (On my computer I set the speed to 0.2)

Part 3 – Running the Residual Reduction

- From the Tools pulldown select “Reduce Residuals”

- In the RRA Tool Dialog select Settings > Load Settings

- Open the file Visual3d_Setup_RRA.xml

- Adjust the “Time range to Process” to the desired start and end time

- Select the Actuators and External Loads Tab

- In the “External Loads Section” change the “External loads specification file” to Visual3d_SIMM_input_grf.xml (Note: This step is new for OpenSim 2.3 and higher)

- Click RUN to start RRA

- Click Close to close the RRA Dialog Tool

Part 4 – Running Computed Muscle Control

- From the Tools pulldown select “Compute Muscle Control “

- In the CMC Tool Dialog select Settings > Load Settings

- Open the file Visual3d_Setup_CMC.xml

- Adjust the “Time range to Process” to the desired start and end time

- Select the Actuators and External Loads Tab

- In the “External Loads Section” change the “External loads specification file” to Visual3d_SIMM_input_grf.xml (Note: This step is new for OpenSim 2.3 and higher)

- Click RUN to start CMC

- Click Close to close the CMC Tool Dialog

Appendix (Creating the v3d_lab)

For example if a lab was set up such that the person walked in the –X direction, +Z was vertical, and +Y was to the subjects left. (See Appendix for an example of Visual3d mdh script could be used to create a compatible virtual lab.

!

! Segment v3d_lab

!

HYBRID_SEGMENT

! /CALIBRATION_FILE=

/TYPE=Visual_3D

/NAME=v3d_lab

/REFERENCE_OBJECT=Medial Distal+Lateral Distal+Medial Proximal+Lateral Proximal+Proximal Joint+Distal Joint+Additional Object+Additional Plane+Distal Radius+Proximal Radius

/REFERENCE_OBJECT_NAMES=v3d_lab_med_distal+v3d_lab_lat_distal+v3d_lab_med_prox+v3d_lab_lat_prox++++Medial+v3d_lab_Distal_Radius+v3d_lab_Proximal_Radius

/REFERENCE_OBJECT_TYPES=LANDMARK+LANDMARK+LANDMARK+LANDMARK+++++METRIC+METRIC

/USE_CAL_TARGETS_FOR_TRACKING=FALSE

/TRACKING_TYPES=LANDMARK+LANDMARK+LANDMARK+LANDMARK

/TRACKING_NAMES=v3d_lab_lat_prox+v3d_lab_med_prox+v3d_lab_lat_distal+v3d_lab_med_distal

! /USE_OPTIMAL_TRACKING=TRUE

/KINEMATIC_ONLY=TRUE

! /STATIC_MODEL=FALSE

/GRAPHICS_ROT_X=0

/GRAPHICS_ROT_Y=0

/GRAPHICS_ROT_Z=0

/GRAPHICS_SCALE_X=1

/GRAPHICS_SCALE_Y=1

/GRAPHICS_SCALE_Z=1

/GRAPHICS_TRANSLATE_X=0

/GRAPHICS_TRANSLATE_Y=0

/GRAPHICS_TRANSLATE_Z=0

! /GRAPHICS_UNIT_SCALE=1.000000

/MASS=0.000000

! /GEOMETRY=

/PROX_TO_CG_AXIAL=0.5*v3d_lab_SEG_LENGTH

/PROX_TO_CG_ML=0*v3d_lab_SEG_LENGTH

/PROX_TO_CG_AP=0*v3d_lab_SEG_LENGTH

/IXX=0

/IYY=0

/IZZ=0

! /AP_DIRECTION=

! /AXIAL_DIRECTION=

! /OBJFILE=

! /COLFILE=

! /DEPTH=

;

! Landmarks

Add_Landmark

/LANDMARK_NAME=v3d_lab_lat_prox

! /CALIBRATION_FILE=

! /USER_GENERATED=TRUE

! /USE_PERCENTAGE=FALSE

! /CALIBRATION_ONLY=FALSE

! /USE_TARGETS=FALSE

/SEGMENT_NAME=LAB

! /TARGET_TYPES=

! /TARGET_NAMES=

! /MCSX=0.0

/MCSY=0.001

! /MCSZ=0.0

! /LANDMARK_LOCATION=

! /REFERENCE_LOCATION_NAME=

! /REFERENCE_LOCATION_TYPE=

! /USE_REFERENCE_LOCATION=FALSE

;

Add_Landmark

/LANDMARK_NAME=v3d_lab_med_prox

! /CALIBRATION_FILE=

! /USER_GENERATED=TRUE

! /USE_PERCENTAGE=FALSE

! /CALIBRATION_ONLY=FALSE

! /USE_TARGETS=FALSE

/SEGMENT_NAME=LAB

! /TARGET_TYPES=

! /TARGET_NAMES=

! /MCSX=0.0

/MCSY=-0.001

! /MCSZ=0.0

! /LANDMARK_LOCATION=

! /REFERENCE_LOCATION_NAME=

! /REFERENCE_LOCATION_TYPE=

! /USE_REFERENCE_LOCATION=FALSE

;

Add_Landmark

/LANDMARK_NAME=v3d_lab_lat_distal

! /CALIBRATION_FILE=

! /USER_GENERATED=TRUE

! /USE_PERCENTAGE=FALSE

! /CALIBRATION_ONLY=FALSE

! /USE_TARGETS=FALSE

/SEGMENT_NAME=LAB

! /TARGET_TYPES=

! /TARGET_NAMES=

! /MCSX=0.0

/MCSY=0.001

/MCSZ=-0.001

! /LANDMARK_LOCATION=

! /REFERENCE_LOCATION_NAME=

! /REFERENCE_LOCATION_TYPE=

! /USE_REFERENCE_LOCATION=FALSE

;

Add_Landmark

/LANDMARK_NAME=v3d_lab_med_distal

! /CALIBRATION_FILE=

! /USER_GENERATED=TRUE

! /USE_PERCENTAGE=FALSE

! /CALIBRATION_ONLY=FALSE

! /USE_TARGETS=FALSE

/SEGMENT_NAME=LAB

! /TARGET_TYPES=

! /TARGET_NAMES=

! /MCSX=0.0

/MCSY=-0.001

/MCSZ=-0.001

! /LANDMARK_LOCATION=

! /REFERENCE_LOCATION_NAME=

! /REFERENCE_LOCATION_TYPE=

! /USE_REFERENCE_LOCATION=FALSE

;